The Project Gutenberg EBook of Manufacturing Cost Data on Artificial Ice, by

Otto Luhr and Herman Friedl

This eBook is for the use of anyone anywhere at no cost and with

almost no restrictions whatsoever.  You may copy it, give it away or

re-use it under the terms of the Project Gutenberg License included

with this eBook or online at www.gutenberg.net

Title: Manufacturing Cost Data on Artificial Ice

Author: Otto Luhr

Herman Friedl

Release Date: July 11, 2009 [EBook #29375]

Language: English

Character set encoding: ASCII

*** START OF THIS PROJECT GUTENBERG EBOOK MFG. COST DATA--ARTIFICIAL ICE ***

Produced by Don Kostuch

Manufacturing Cost Data

ON

Artificial

Ice

MADE IN

ACCORDANCE WITH

OTTO LUHR

CONSULTING ENGINEER

&

HERMAN  FRIEDL

ARCHITECT

ICE MAKING SYSTEM

PATENT APPLIED FOR

154 WEST RANDOLPH STREET

CHICAGO, ILL.

Ice for Commercial Purposes

ICE for commercial purposes is obtained in two ways: either by cutting during the winter time from our lakes and rivers and storing in large Ice Storage Houses located alongside, or by freezing pure clean water by means of artificial refrigeration.

All authorities are agreed that artificial ice is more sanitary than natural ice and it is only a matter of time when the use of natural ice will be prohibited except in special cases when the purity of its source of supply is beyond doubt.

Our improved method of making artificial ice will cut the labor cost down to the minimum and will enable the manufacturer to profitably sell artificial ice at the price natural ice can be harvested. The logical result thereof will be the building of a large number of modern ice plants all over the country to supply the market with artificial ice in place of the present natural ice.

We do not claim any wonders for our system but believe that the following points of advantage will convince any practical ice manufacturer that the labor cost has been cut in two.

First. We pull a complete row of the full width of tank at one time.

Second. Our air supply is permanently connected to the cans and the supply to each can can be regulated, if required.

Third. We have a continuous air supply to the cans during freezing as well as during thawing, dumping and filling. Our air supply never ceases.

Fourth. Our air is automatically cooled down to the temperature of the brine in the tank thereby eliminating all possibility of moisture in the air pipes.

Fifth. Our cans are held in a solid frame of steel work and are connected to the crane from the time the cans are pulled until they are put back into the tanks, thereby doubling the life of the cans.

We give herewith data covering the cost of manufacturing ice and will guarantee that under reasonably fair management the number of men required will not be exceeded.

Do not fail to carefully analyze the following cost data. They may seem extremely low but a thorough study of our system will prove them to be very conservative.

[1]

NUMBER ONE

Manufacturing Costs Per Ton of Ice

Using Electric Power at Present Chicago

Rates for Power and Labor

Capacity of plant, 240 tons of ice per day, using 2692 cans of 400-lb. capacity.

18000-ton storage house.

Average current requirement for freezing one ton of ice, including storage cooling and all auxiliaries, 55 K. W. hours.

Average cost per K. W. hour, .9 cent.

Current cost per ton of ice, 55 x .9, equals 49.6 cents.

Assuming one month's shut-down for inspection and repairs, the total output of 240 tons of ice for 333 days amounts to 79,920 tons, or roughly speaking 80,000 tons of ice.

Adding 1/2 cent per ton of ice for the required heating, the total power

cost of  making 80,000 tons of ice is (80,000 x .50) 	$ 40,000.00

ENGINE ROOM LABOR COST:

1 chief engineer per day 	$ 10.00

3 engineers per day 	$ 8.00

Total per day 	$ 34.00

365 days at $34.00 equals	$ 12,410.00

or  12410 / 80000 = 15.62 cents per ton

[2]

ICE PLANT LABOR COST:

3 men pulling ice and setting it up in store-room.	per day $ 6.00

3 men in store-room	per day $ 6.00

1 shipping clerk	per day $ 8.00

Total labor	per day $ 44.00

365 days at $44.00 equals		 16,060.00

For filling the winter storage house and

taking the ice out of it will require

3 additional men for five months,

equals 150 days x $18.00, equals 	$ 2,700.00

Total Ice Plant Labor Cost Equals 	$18,760.00

or  18670 / 80000 = 23.46 cents per ton

240 tons of ice equal 1200---400-lb. cans. As 24 cans are pulled at one time it requires 1200 / 24 = 50 pulls per day, or one pull every 29 minutes. The ice-puller has therefore ample time to set up all ice pulled in storage house as directed.

Cost of Ammonia at 	2 cent per ton		$ 1,600.00

Cost of Oil and Waste at 	2 cent per ton		$ 1,600.00

Cost of Water at 	3 cent per ton 		$ 2,400.00

Cost of Salt at 	72 cent per ton 		$ 400.00

Plant Maintenance and Repairs		$ 3,500.00

or  3500 / 80000 = 4.37 cent per ton

OFFICE EXPENSES:

1 Manager and Salesman, per year 	$ 5,000.00

1 Bookkeeper, per year	$ 2,400.00

Stationery, Telephone, etc 	$ 600.00

Total Cost 		$ 8,000.00

or  8000  / 80000 = 10 cent per ton

[3]

OVERHEAD CHARGES:

8 per cent interest on $350,000.00 investment	$ 28,000.00

8 per cent interest on value of land ($20,000.00)	$ 1,600.00

8 per cent interest on $10,000.00 working capital	$ 800.00

3 per cent depreciation on $350,000.00	$ 10,500.00

Insurance (estimated)	$ 1,500.00

Taxes (estimated) 	 $ 3,500.00

Total		 $ 45,900.00

or 45900  / 80000 = 57.375 cent per ton

Total Expense		$134,570.00

or 134570 / 80000 = $1.68.215 per ton

Divided as follows:--

Manufacturing cost including office expense	$ 1.10.840

Overhead charges 	$ 0.57.375

ICE SALES ASSUMPTIONS:

| Month | Ice produced per day | Ice sold direct per day | Ice stored per day | Ice Sold from storage daily | Total Ice stored in 30 days |
| --- | --- | --- | --- | --- | --- |
| January | 240 | 65 | 175 |  | 5250 |
| February | 240 | 65 | 175 |  | 5250 |
| March | 240 | 115 | 125 |  | 3750 |
| April | 240 | 165 | 75 |  | 2250 |
| May | 240 | 300 |  | 60 |  |
| June | 240 | 400 |  | 160 |  |
| July | 240 | 400 |  | 160 |  |
| August | 240 | 400 |  | 160 |  |
| September | 240 | 350 |  | 110 |  |
| October | 240 | 200 | 40 |  | 1200 |
| November | 240 | 140 | 100 |  | 3000 |
| December | None | 65 |  | 65 |  |

Tons	20700

Less Tons	1950

Total Tons	18750

During the month of December, the Ice Plant will be shut down for overhauling and repairs, and part of the ice stored during November will be sold in December, therefore, requiring a total storage capacity of 18,750 tons, of which 750 tons will be stored in the ante-room and 18,000 tons will be stored in the big winter storage.

[4]

NUMBER TWO

Manufacturing Costs Per Ton of Ice Using

Electric Power at Present Chicago Rates for Power and Labor

240 TON CAPACITY PER DAY

No Storage House for Surplus Ice

ICE SALES ASSUMPTIONS:

Tons per day  	Total Tons

January	65 	1,950

February	65 	1,950

March 	115 	3,450

April	165	 4,950

May	240 	7,200

June 	240 	7,200

July	 240 	7,200

August 	240 	7,200

September.	240 	7,200

October 	 200 	6,000

November	140 	4,200

December	65 	1,950

Total output tons 	60,450

NOTE--These sales can only be realized if the dealer has at least 18,000 tons of natural ice on hand to enable him to take care of the family trade during the hot months.

If no large supply of natural ice is on hand hardly 50,000 tons can be sold, thereby increasing the cost per ton considerably.

POWER COST:

Due to numerous starting and stopping of compressor during the slack months the maximeter charges will be higher and therefore it must be assumed that 60 K. W. hours will be required per ton of ice instead of 55 K. W. hours for continuous consumption.

60 K. W. hours per ton of ice at .9 cent per K. W. hour equals 54 cents per ton. Adding 1/2 cent per ton for the required heating the power cost for making 60,450 tons of ice equals 60,450 x 54.5 cents, equals	$ 32,945.25

[ 5 ]

ENGINE ROOM LABOR COST:

1 chief engineer per day	$ 10.00

3 engineers per day	$ 8.00

Total per day 	$ 34.00

365 days at $34.00 equals			$ 12,410.00

or  12410 / 60450 = 20.54 cent per ton of ice

ICE PLANT LABOR COST:

(Using present method of pulling ice)

May, June, July, August, September and October require:

6 ice pullers per day 	$ 6.00

3 air men per day 	$ 6.00

6 storage house men per day 	$ 6.00

Total per day 	$ 90.00

184 days at $90.00 equals		$ 16,560.00

March, April and November require:

6 pullers per day 	$ 6.00

4 storage house men per day 	$ 6.00

Total per day 	$ 60.00

91 days at $60.00 equals		$ 5,460.00

December, January and February require:

3 pullers per day 	$ 6.00

3 storage house men per day 	$ 6.00

Total per day 	$ 36.00

92 days at $36.00 equals		$ 3,312.00

1 shipping clerk per day 	$ 8.00

330 days x 8 equals		$ 2,640.00

Total Labor Cost		$ 27,972.00

or  27972 / 60450 = 46.27 cent per ton

Cost of Ammonia at 	2 cent per ton		$ 1,209.00

Cost of Oil and Waste at 	2 cent per ton		$ 1,209.00

Cost of Water at 	3 cent per ton		$ 1,813.50

Cost of Salt at 	1/2 cent per ton		$ 302.25

Plant Maintenance and Repairs		$ 2,800.00

or 2800  / 60450 = 4.63 cent per ton

[6]

OFFICE EXPENSE:

1 Manager and Salesman per year	$ 5,000.00

1 Bookkeeper per year	$ 2,400.00

Stationery, Telephone, etc	$ 600.00

Total Cost		$ 8,000.00

or 8000 / 60450 = 13.23 cent per ton

OVERHEAD CHARGES:

8 per cent Interest on $280,000.00 investment	$ 22,400.00

8 per cent Interest on value of land ($12,000.00)	$ 960.00

8 per cent interest on $8,000.00 working capita	$ 640.00

3 per cent depreciation on $280,000.00	$ 8,400.00

Insurance (estimated)	$ 1,200.00

Taxes (estimated)	$ 2,500.00

Total Overhead Charge		 36,100.00

or 36100  / 60450 = 69.72 cent per ton

Total Expense		$124,961.00

or 124961 / 60450 = $2.06.72 per ton

NOTE--If the LUHR & FRIEDL ICE MAKING SYSTEM is used, the Ice Plant Labor Cost will be as follows:

May, June, July, August, September and October require:

3 ice pullers	 per day $ 6.00

3 storage house men 	per day $ 6.00

Total	per day $ 36.00

184 days at $ 36.00 equals		$ 6,624.00

March, April and November require:

3 ice pullers 	per day $ 6.00

2 storage house men 	per day $ 6.00

Total per day 	$ 30.00

91 days at $ 30.00 equals		$ 2,730.00

December, January and February require:

3 ice pullers	per day $ 6.00

1 storage house man 	per day $ 6.00

Total per day 	$ 24.00

92 days at $ 24.00 equals		$ 2,208.00

1 shipping clerk 	per day $ 8.00

330 days x 8 equals.		$ 2,640.00

Total Labor Cost.		$ 14,202.00

or 14202  / 60450 = 23.49 cent per ton

compared to 46.27 cent per ton, A SAVING OF 22.78 CENT PER TON.

[7]

Typical Design of a 160 Ton Steam Driven Ice Plant.

Interior Details

[8]

Typical Design of a 160 Ton Steam Driven Ice Plant.

Exterior Cross Section

In connection with

Otto Luhr

Consulting Engineer

& Herman Fridel

Architect

Ice Making System

Patent Applied For

[9]

NUMBER THREE

Manufacturing Costs Per Ton of Ice Using

Steam Power at Medium-Sized-Town Rates for Labor

160-ton capacity per day.

1,728--400-lb. cans.

333 days continuous full output.

12,000-ton storage house.

COST OF POWER:

A modern, highly efficient and economical steam driven high speed compressor plant must be installed so as to get the maximum power out of coal. The boiler room will contain two 250-H. P. water-tube boilers with automatic stokers and coal bin overhead holding two weeks' supply of coal. Steam pressure 175 lbs. As the firing of the boilers is automatic and requires practically no work on the part of the engineers, no firemen are needed. Ashes will also be removed automatically. The engine room equipment will consist of two 175-ton high speed compressors, direct connected to two Simple Condensing Una-flow Engines; also two generators, two cooling tower water pumps, two air compressors, switchboard, etc. All to be equipped with the latest labor and power-saving devices.

Equipped as above, 25 tons of refrigeration can be easily obtained from one ton of ordinary 12,500 B T U coal. 1.8 ton of refrigeration is required to produce one ton of ice including the required cooling of storage house.

Therefore the power cost of making one ton of ice with coal at $5.00 per ton equals $5.00 divided by  25/1.8 = 37 cent. (One cent per ton of ice is added for heating of dipping tank water.)

Assuming one month's shut-down for inspection and repairs, the total output of 160 tons of ice for 333 days amounts to 53,280 tons of ice.

The total power cost of making 53,280 tons of ice is therefore,

53,280 x 37 cent 	$ 19,713.60

[10]

ENGINE ROOM AND ICE PLANT LABOR COST:

1 chief engineer 	per day $ 8.00

3 engineers	 per day $ 6.00

1 shipping clerk	 per day $ 6.00

3 men in Storage House	 per day $ 4.00

Total	 per day $ 44.00

365 days at $44.00 per day equals 		$ 16,060.00

Additional labor cost for putting 12,000 tons into winter

storage and taking out at $4.00 per day 		$ 1,200.00

Total Labor Cost		$ 17,260.00

or 17260 / 53280 = 32.4 cent per ton

Engineers will do their own firing of boilers and will pull all the ice. One pull required every 43 minutes.

OFFICE EXPENSE:

1 Office Man (Manager and Bookkeeper)		$ 3,000.00

Stationery, Telephone, etc. (per year)		$ 300.00

Total Office Expense			$ 3,300.00

or 3300 / 53280 = 6.2 cent per ton of ice

Cost of Ammonia at 2 cent per ton			$ 1,065.60

Cost of Oil and Waste at 2 cent per ton			$ 1,065.60

Cost of Water at 3 cent per ton			$ 1,598.40

Cost of Salt at 1/2 cent per ton			$ 266.40

Plant Maintenance and Repairs			$ 2,200.00

or 2200 / 53280 = 4.1 cent per ton

[11]

OVERHEAD CHARGES:

8 per cent interest on $220,000.00 investment equals	$ 17,600.00

8 per cent interest on value of land ($10,000.00) 	$ 800.00

8 per cent interest on working capital ($7,500.00)	$ 600.00

3 per cent depreciation on $220,000.00	$ 6,600.00

Insurance (estimated)	$ 1,000.00

Taxes (estimated)	$ 2,000.00

Total overhead charges 		$ 28,600.00

or 28600 / 53280 = 53.7 cent per ton

Total Expense		$ 75,069.60

or 75069.60 / 53280 = $ 1.409 per ton

Divided as follows:

Overhead charges		$ 0.53.7

Manufacturing Cost (total)		$ 0.87.2

[12]

NUMBER FOUR

Manufacturing Costs Per Ton of Ice Using Steam Power at

Medium-Sized-Town Rates for Labor

100-ton capacity per day.

1,080--400-1b. cans.

333 days continuous full output.

7,600-ton Storage House.

COST OF POWER:

A modern, highly efficient and economical steam driven high speed compressor plant must be installed so as to get the maximum power out of coal. The boiler-room will contain two 200-H. P. water-tube boilers with automatic stokers and coal bin overhead holding two weeks' supply of coal. Steam pressure 175 lbs. As the firing of the boilers is automatic and requires practically no work on the part of the engineers, no firemen will be needed. Ashes will also be automatically removed. The engine room equipment will consist of two 100-ton high speed compressors, direct connected to two Simple Condensing Unaflow Engines; also two generators, two cooling tower pumps, two air compressors, switchboard, etc. All to be equipped with the latest labor and power-saving devices.

Equipped as above, 25 tons of refrigeration can be easily obtained from one ton of ordinary 12500 B T U coal. 1.8 tons of refrigeration is required to produce one ton of ice, including the cooling of the storage house.

Therefore, the power cost of making one ton of ice with coal at $5.00 per ton equals $5.00 divided by 25/1.8 = 37 cent. (One cent per ton of ice is added for heating of dipping-tank water.

Assuming one month's shut down for inspection and repairs, the total output of 100 tons of ice for 333 days amounts to 33,300 tons of ice.

The total power cost of making 33,300 tons of ice is therefore,

33,300 x 37 cent, equals	$ 12,321.00

[13]

ENGINE ROOM AND ICE PLANT LABOR COST:

1 Chief Engineer per day 	$ 8.00

3 Engineers per day 	$ 6.00

2 Storage House Men per day	 $ 4.00

Total per day	$ 34.00

Total 365 days at $34.00 per day		$ 12,410.00

Additional labor cost for putting 7,500 tons into winter

storage and taking out at $4.00 per day		$ 750.00

Total labor cost		$ 13,160.00

or 13160 / 33300 = 39.52 cent per ton

Engineer will do his own firing of boilers and will pull all the ice and set it up in ante room if required. One pull required every 70 minutes.

Chief Engineer will act as shipping clerk.

OFFICE EXPENSE:

1 Office Man (Manager and Bookkeeper)		 $ 3,000.00

Stationery, Telephone, etc. (per year)		 $ 300.00

Total Office Expense			$ 3,300.00

or 3300  / 33300 = 9.9 cent per ton

Cost of Ammonia at 	2 cent per ton			$ 666.00

Cost of Oil and Waste at 	2 cent per ton			$ 666.00

Cost of Water at 	3 cent per ton			$ 999.00

Cost of Salt at 	1/2 cent per ton			$ 166.50

Plant Maintenance and Repairs			$ 1,500.00

or 1500 / 33300 = 4.5 cent per ton

[14]

OVERHEAD CHARGES:

8 per cent interest on $150,000.00 investment	$ 12,000.00

8 per cent interest on value of land ($7,000.00)	$ 560.00

8 per cent interest on $5,000.00 working capital	$ 400.00

3 per cent depreciation on $150,000.00	$ 4,500.00

Insurance (estimated)	$ 700.00

Taxes (estimated)	$ 1,360.00

Total overhead charges		$ 19,520.00

or 19520  / 33300 == 68.7 cent per ton

Total Expense		$ 52,298.50

or 52298.50 / 33300 = $1.57 per ton

Divided as follows:

Overhead charges		68.7 cent

Manufacturing Cost		98.3 cent

[15]

OTTO  LUHR

CONSULTING ENGINEER

&

HERMAN FRIEDL

ARCHITECT

ICE MAKING SYSTEM

154 W. RANDOLPH STREET, CHICAGO

[Transcriber's Note]

I found this document and the attached papers and photographs among my father's papers. I offer it as an insight into the finances and structure of business and trades in the early 1900's.

There are no dates included in this document but a Google search of "Otto Luhr" produced these items:

-- Mechanical and Refrigerating Engineer's Handy Book; Otto Luhr; 1913.

-- Automatic refrigerating liquid feeder and regulator;

United States Patent 1725875; 8/27/1929.

-- Refrigerator car; United States Patent 1642882; 9/20/1927.

Since the title page states "Patent Applied For", this document was probably published around 1925.

Note the prices quoted for materials and labor:

Coal, $5.00 a ton.  [In 2009, about $100/ton, down from $300 in 2008.]

Unskilled Labor, $6.00/day; that's DAY, not HOUR.

Skilled Labor, $8 to $10/day

Electricity, $0.009/KWH  [my latest bill (in 2009) was $0.1317/KWH]

Note the job titles in the attached documents: Barnmen, Washers, Blacksmiths

Finally, the optimistic tone of the document contrasts with the decline of the ice business in the 1940's, fifteen years later. I remember the ice deliveries and the weight sign my mother put in the window before we got our first mechanical refrigerator after World War II.

[End Transcriber's Note]

DETROIT CREAMERY COMPANY

ORGANIZATION

1 -- Board of Directors

2 -- Operating Committee

Harry A. McDonald 	President

Nelson J. Dessert	Vice president

Carl F. Siclaff	Vice president

Harry J. Weigand	Treasurer & Comptroller

Jerome H. Remick	Ice Cream Sales & Service

J. Harry Brickley 	Retail Milk Sales

Oliver G. Spaulding 	Legal Department

Richard L. Baire	Advertising

Frank McVeigh	Purchasing Department

Ben F. Taylor	Ice Cream Production

Ben F. Taylor 	Ice Cream Delivery

Edward C. Krahl 	Henry St. Production

Doc Grayson 	Laboratory

John Kostuch	Plant Engineer - Maintenance

John Kostuch 	Power & Refrigeration

J. Harry Watson 	Transportation

J. Harry Watson 	Shops

H. Terry Snowday	Wholesale Milk Sales

Carl O. Tuttle 	Butter Department

Tom Wood 	Credit & Collections

J. McWilliams	Detroit Creamery Farms

TREASURER & COMPTROLLER

Harry J. Weigand

Accounting - Detroit Creamery & Subsidiaries

Loans & Contracts - Detroit Creamery & Subsidiaries

Appropriations - Detroit Creamery & Subsidiaries

Banks  - Detroit Creamery & Subsidiaries

Account Dept Personnel - Detroit Creamery & Subsidiaries

Credits & Collections

Corporate Records

Purchasing Department

Legal Department

PLANT ENGINEERING--MAINTAINANCE

POWER and REFRIGERATION

John Kostuch (Chief Engineer)

Paul Culver (Consulting)

Norman Mitehell (Technical) (Advisory) (Dept. Head)

HENRY STREET (MAINTAINANCE)

James Crunnley (In Charge)

(a) Electrical & General (Ray Casson)

(b) Conveyors, Bottle Washers, Fillers, Cappers (Howard Strauss)

(c) All other Machinery (Assign Mechanics)

HENRY STREET (POWER & REFRIGERATION)

Harry Hollenbeck (In Charge)

(a) Engineers

(b) Firemen

MAIN PLANT (MAINTAINANCE) (POWER & REFRIGERATION)

John Kostuch (In Charge)

REC. STATIONS & MFG. PLANTS

John Kostuch (Chief)

Elmer DeWitt(Asst)

Frank Mortimer (Mech)

C. S. McBride (Production Dept.)

SUBSIDIARY COMPANIES

John Kostuch (Chief)

Paul Culver--Norman Mitchell--Dept. Head

MACHINE SHOP (MAIN PLANT)

John Kostuch (In Charge)

TRANSPORTATION & SHOPS

J. Harry Watson

Garages 	Detroit Subs. (Advisory)

Auto Shops	Detroit (Met. Area)

Subs. (Advisory)

Paint Shops	Detroit & Subs.

Electrical Shops 	Detroit Subs. (Advisory)

Carpenter Shops	Detroit & Sub. (Advisory)

Stables 	Detroit (Advisory)

Barnmen 	Sub. (Advisory)

Washers

Blacksmiths

Wagon Shops 	Detroit & Subs

Harness Shops 	Detroit & Subs.

Plumbing Shops 	Detroit

Sign Shop 	Detroit & Subs.

Tin Shop 	Detroit & Subs.

Special Delivery

and Trucking 	Detroit (Main)

Branch Trucking

Special Trucking

End of the Project Gutenberg EBook of Manufacturing Cost Data on Artificial

Ice, by Otto Luhr and Herman Friedl

*** END OF THIS PROJECT GUTENBERG EBOOK MFG. COST DATA--ARTIFICIAL ICE ***

***** This file should be named 29375-doc.doc or 29375-doc.zip *****

This and all associated files of various formats will be found in:

http://www.gutenberg.org/2/9/3/7/29375/

Produced by Don Kostuch

Updated editions will replace the previous one--the old editions

will be renamed.

Creating the works from public domain print editions means that no

one owns a United States copyright in these works, so the Foundation

(and you!) can copy and distribute it in the United States without

permission and without paying copyright royalties.  Special rules,

set forth in the General Terms of Use part of this license, apply to

copying and distributing Project Gutenberg-tm electronic works to

protect the PROJECT GUTENBERG-tm concept and trademark.  Project

Gutenberg is a registered trademark, and may not be used if you

charge for the eBooks, unless you receive specific permission.  If you

do not charge anything for copies of this eBook, complying with the

rules is very easy.  You may use this eBook for nearly any purpose

such as creation of derivative works, reports, performances and

research.  They may be modified and printed and given away--you may do

practically ANYTHING with public domain eBooks.  Redistribution is

subject to the trademark license, especially commercial

redistribution.

*** START: FULL LICENSE ***

THE FULL PROJECT GUTENBERG LICENSE

PLEASE READ THIS BEFORE YOU DISTRIBUTE OR USE THIS WORK

To protect the Project Gutenberg-tm mission of promoting the free

distribution of electronic works, by using or distributing this work

(or any other work associated in any way with the phrase "Project

Gutenberg"), you agree to comply with all the terms of the Full Project

Gutenberg-tm License (available with this file or online at

http://gutenberg.net/license).

Section 1.  General Terms of Use and Redistributing Project Gutenberg-tm

electronic works

1.A.  By reading or using any part of this Project Gutenberg-tm

electronic work, you indicate that you have read, understand, agree to

and accept all the terms of this license and intellectual property

(trademark/copyright) agreement.  If you do not agree to abide by all

the terms of this agreement, you must cease using and return or destroy

all copies of Project Gutenberg-tm electronic works in your possession.

If you paid a fee for obtaining a copy of or access to a Project

Gutenberg-tm electronic work and you do not agree to be bound by the

terms of this agreement, you may obtain a refund from the person or

entity to whom you paid the fee as set forth in paragraph 1.E.8.

1.B.  "Project Gutenberg" is a registered trademark.  It may only be

used on or associated in any way with an electronic work by people who

agree to be bound by the terms of this agreement.  There are a few

things that you can do with most Project Gutenberg-tm electronic works

even without complying with the full terms of this agreement.  See

paragraph 1.C below.  There are a lot of things you can do with Project

Gutenberg-tm electronic works if you follow the terms of this agreement

and help preserve free future access to Project Gutenberg-tm electronic

works.  See paragraph 1.E below.

1.C.  The Project Gutenberg Literary Archive Foundation ("the Foundation"

or PGLAF), owns a compilation copyright in the collection of Project

Gutenberg-tm electronic works.  Nearly all the individual works in the

collection are in the public domain in the United States.  If an

individual work is in the public domain in the United States and you are

located in the United States, we do not claim a right to prevent you from

copying, distributing, performing, displaying or creating derivative

works based on the work as long as all references to Project Gutenberg

are removed.  Of course, we hope that you will support the Project

Gutenberg-tm mission of promoting free access to electronic works by

freely sharing Project Gutenberg-tm works in compliance with the terms of

this agreement for keeping the Project Gutenberg-tm name associated with

the work.  You can easily comply with the terms of this agreement by

keeping this work in the same format with its attached full Project

Gutenberg-tm License when you share it without charge with others.

1.D.  The copyright laws of the place where you are located also govern

what you can do with this work.  Copyright laws in most countries are in

a constant state of change.  If you are outside the United States, check

the laws of your country in addition to the terms of this agreement

before downloading, copying, displaying, performing, distributing or

creating derivative works based on this work or any other Project

Gutenberg-tm work.  The Foundation makes no representations concerning

the copyright status of any work in any country outside the United

States.

1.E.  Unless you have removed all references to Project Gutenberg:

1.E.1.  The following sentence, with active links to, or other immediate

access to, the full Project Gutenberg-tm License must appear prominently

whenever any copy of a Project Gutenberg-tm work (any work on which the

phrase "Project Gutenberg" appears, or with which the phrase "Project

Gutenberg" is associated) is accessed, displayed, performed, viewed,

copied or distributed:

This eBook is for the use of anyone anywhere at no cost and with

almost no restrictions whatsoever.  You may copy it, give it away or

re-use it under the terms of the Project Gutenberg License included

with this eBook or online at www.gutenberg.net

1.E.2.  If an individual Project Gutenberg-tm electronic work is derived

from the public domain (does not contain a notice indicating that it is

posted with permission of the copyright holder), the work can be copied

and distributed to anyone in the United States without paying any fees

or charges.  If you are redistributing or providing access to a work

with the phrase "Project Gutenberg" associated with or appearing on the

work, you must comply either with the requirements of paragraphs 1.E.1

through 1.E.7 or obtain permission for the use of the work and the

Project Gutenberg-tm trademark as set forth in paragraphs 1.E.8 or

1.E.9.

1.E.3.  If an individual Project Gutenberg-tm electronic work is posted

with the permission of the copyright holder, your use and distribution

must comply with both paragraphs 1.E.1 through 1.E.7 and any additional

terms imposed by the copyright holder.  Additional terms will be linked

to the Project Gutenberg-tm License for all works posted with the

permission of the copyright holder found at the beginning of this work.

1.E.4.  Do not unlink or detach or remove the full Project Gutenberg-tm

License terms from this work, or any files containing a part of this

work or any other work associated with Project Gutenberg-tm.

1.E.5.  Do not copy, display, perform, distribute or redistribute this

electronic work, or any part of this electronic work, without

prominently displaying the sentence set forth in paragraph 1.E.1 with

active links or immediate access to the full terms of the Project

Gutenberg-tm License.

1.E.6.  You may convert to and distribute this work in any binary,

compressed, marked up, nonproprietary or proprietary form, including any

word processing or hypertext form.  However, if you provide access to or

distribute copies of a Project Gutenberg-tm work in a format other than

"Plain Vanilla ASCII" or other format used in the official version

posted on the official Project Gutenberg-tm web site (www.gutenberg.net),

you must, at no additional cost, fee or expense to the user, provide a

copy, a means of exporting a copy, or a means of obtaining a copy upon

request, of the work in its original "Plain Vanilla ASCII" or other

form.  Any alternate format must include the full Project Gutenberg-tm

License as specified in paragraph 1.E.1.

1.E.7.  Do not charge a fee for access to, viewing, displaying,

performing, copying or distributing any Project Gutenberg-tm works

unless you comply with paragraph 1.E.8 or 1.E.9.

1.E.8.  You may charge a reasonable fee for copies of or providing

access to or distributing Project Gutenberg-tm electronic works provided

that

- You pay a royalty fee of 20% of the gross profits you derive from

the use of Project Gutenberg-tm works calculated using the method

you already use to calculate your applicable taxes.  The fee is

owed to the owner of the Project Gutenberg-tm trademark, but he

has agreed to donate royalties under this paragraph to the

Project Gutenberg Literary Archive Foundation.  Royalty payments

must be paid within 60 days following each date on which you

prepare (or are legally required to prepare) your periodic tax

returns.  Royalty payments should be clearly marked as such and

sent to the Project Gutenberg Literary Archive Foundation at the

address specified in Section 4, "Information about donations to

the Project Gutenberg Literary Archive Foundation."

- You provide a full refund of any money paid by a user who notifies

you in writing (or by e-mail) within 30 days of receipt that s/he

does not agree to the terms of the full Project Gutenberg-tm

License.  You must require such a user to return or

destroy all copies of the works possessed in a physical medium

and discontinue all use of and all access to other copies of

Project Gutenberg-tm works.

- You provide, in accordance with paragraph 1.F.3, a full refund of any

money paid for a work or a replacement copy, if a defect in the

electronic work is discovered and reported to you within 90 days

of receipt of the work.

- You comply with all other terms of this agreement for free

distribution of Project Gutenberg-tm works.

1.E.9.  If you wish to charge a fee or distribute a Project Gutenberg-tm

electronic work or group of works on different terms than are set

forth in this agreement, you must obtain permission in writing from

both the Project Gutenberg Literary Archive Foundation and Michael

Hart, the owner of the Project Gutenberg-tm trademark.  Contact the

Foundation as set forth in Section 3 below.

1.F.

1.F.1.  Project Gutenberg volunteers and employees expend considerable

effort to identify, do copyright research on, transcribe and proofread

public domain works in creating the Project Gutenberg-tm

collection.  Despite these efforts, Project Gutenberg-tm electronic

works, and the medium on which they may be stored, may contain

"Defects," such as, but not limited to, incomplete, inaccurate or

corrupt data, transcription errors, a copyright or other intellectual

property infringement, a defective or damaged disk or other medium, a

computer virus, or computer codes that damage or cannot be read by

your equipment.

1.F.2.  LIMITED WARRANTY, DISCLAIMER OF DAMAGES - Except for the "Right

of Replacement or Refund" described in paragraph 1.F.3, the Project

Gutenberg Literary Archive Foundation, the owner of the Project

Gutenberg-tm trademark, and any other party distributing a Project

Gutenberg-tm electronic work under this agreement, disclaim all

liability to you for damages, costs and expenses, including legal

fees.  YOU AGREE THAT YOU HAVE NO REMEDIES FOR NEGLIGENCE, STRICT

LIABILITY, BREACH OF WARRANTY OR BREACH OF CONTRACT EXCEPT THOSE

PROVIDED IN PARAGRAPH F3.  YOU AGREE THAT THE FOUNDATION, THE

TRADEMARK OWNER, AND ANY DISTRIBUTOR UNDER THIS AGREEMENT WILL NOT BE

LIABLE TO YOU FOR ACTUAL, DIRECT, INDIRECT, CONSEQUENTIAL, PUNITIVE OR

INCIDENTAL DAMAGES EVEN IF YOU GIVE NOTICE OF THE POSSIBILITY OF SUCH

DAMAGE.

1.F.3.  LIMITED RIGHT OF REPLACEMENT OR REFUND - If you discover a

defect in this electronic work within 90 days of receiving it, you can

receive a refund of the money (if any) you paid for it by sending a

written explanation to the person you received the work from.  If you

received the work on a physical medium, you must return the medium with

your written explanation.  The person or entity that provided you with

the defective work may elect to provide a replacement copy in lieu of a

refund.  If you received the work electronically, the person or entity

providing it to you may choose to give you a second opportunity to

receive the work electronically in lieu of a refund.  If the second copy

is also defective, you may demand a refund in writing without further

opportunities to fix the problem.

1.F.4.  Except for the limited right of replacement or refund set forth

in paragraph 1.F.3, this work is provided to you 'AS-IS' WITH NO OTHER

WARRANTIES OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO

WARRANTIES OF MERCHANTIBILITY OR FITNESS FOR ANY PURPOSE.

1.F.5.  Some states do not allow disclaimers of certain implied

warranties or the exclusion or limitation of certain types of damages.

If any disclaimer or limitation set forth in this agreement violates the

law of the state applicable to this agreement, the agreement shall be

interpreted to make the maximum disclaimer or limitation permitted by

the applicable state law.  The invalidity or unenforceability of any

provision of this agreement shall not void the remaining provisions.

1.F.6.  INDEMNITY - You agree to indemnify and hold the Foundation, the

trademark owner, any agent or employee of the Foundation, anyone

providing copies of Project Gutenberg-tm electronic works in accordance

with this agreement, and any volunteers associated with the production,

promotion and distribution of Project Gutenberg-tm electronic works,

harmless from all liability, costs and expenses, including legal fees,

that arise directly or indirectly from any of the following which you do

or cause to occur: (a) distribution of this or any Project Gutenberg-tm

work, (b) alteration, modification, or additions or deletions to any

Project Gutenberg-tm work, and (c) any Defect you cause.

Section  2.  Information about the Mission of Project Gutenberg-tm

Project Gutenberg-tm is synonymous with the free distribution of

electronic works in formats readable by the widest variety of computers

including obsolete, old, middle-aged and new computers.  It exists

because of the efforts of hundreds of volunteers and donations from

people in all walks of life.

Volunteers and financial support to provide volunteers with the

assistance they need are critical to reaching Project Gutenberg-tm's

goals and ensuring that the Project Gutenberg-tm collection will

remain freely available for generations to come.  In 2001, the Project

Gutenberg Literary Archive Foundation was created to provide a secure

and permanent future for Project Gutenberg-tm and future generations.

To learn more about the Project Gutenberg Literary Archive Foundation

and how your efforts and donations can help, see Sections 3 and 4

and the Foundation web page at http://www.pglaf.org.

Section 3.  Information about the Project Gutenberg Literary Archive

Foundation

The Project Gutenberg Literary Archive Foundation is a non profit

501(c)(3) educational corporation organized under the laws of the

state of Mississippi and granted tax exempt status by the Internal

Revenue Service.  The Foundation's EIN or federal tax identification

number is 64-6221541.  Its 501(c)(3) letter is posted at

http://pglaf.org/fundraising.  Contributions to the Project Gutenberg

Literary Archive Foundation are tax deductible to the full extent

permitted by U.S. federal laws and your state's laws.

The Foundation's principal office is located at 4557 Melan Dr. S.

Fairbanks, AK, 99712., but its volunteers and employees are scattered

throughout numerous locations.  Its business office is located at

809 North 1500 West, Salt Lake City, UT 84116, (801) 596-1887, email

business@pglaf.org.  Email contact links and up to date contact

information can be found at the Foundation's web site and official

page at http://pglaf.org

For additional contact information:

Dr. Gregory B. Newby

Chief Executive and Director

gbnewby@pglaf.org

Section 4.  Information about Donations to the Project Gutenberg

Literary Archive Foundation

Project Gutenberg-tm depends upon and cannot survive without wide

spread public support and donations to carry out its mission of

increasing the number of public domain and licensed works that can be

freely distributed in machine readable form accessible by the widest

array of equipment including outdated equipment.  Many small donations

($1 to $5,000) are particularly important to maintaining tax exempt

status with the IRS.

The Foundation is committed to complying with the laws regulating

charities and charitable donations in all 50 states of the United

States.  Compliance requirements are not uniform and it takes a

considerable effort, much paperwork and many fees to meet and keep up

with these requirements.  We do not solicit donations in locations

where we have not received written confirmation of compliance.  To

SEND DONATIONS or determine the status of compliance for any

particular state visit http://pglaf.org

While we cannot and do not solicit contributions from states where we

have not met the solicitation requirements, we know of no prohibition

against accepting unsolicited donations from donors in such states who

approach us with offers to donate.

International donations are gratefully accepted, but we cannot make

any statements concerning tax treatment of donations received from

outside the United States.  U.S. laws alone swamp our small staff.

Please check the Project Gutenberg Web pages for current donation

methods and addresses.  Donations are accepted in a number of other

ways including including checks, online payments and credit card

donations.  To donate, please visit: http://pglaf.org/donate

Section 5.  General Information About Project Gutenberg-tm electronic

works.

Professor Michael S. Hart is the originator of the Project Gutenberg-tm

concept of a library of electronic works that could be freely shared

with anyone.  For thirty years, he produced and distributed Project

Gutenberg-tm eBooks with only a loose network of volunteer support.

Project Gutenberg-tm eBooks are often created from several printed

editions, all of which are confirmed as Public Domain in the U.S.

unless a copyright notice is included.  Thus, we do not necessarily

keep eBooks in compliance with any particular paper edition.

Most people start at our Web site which has the main PG search facility:

http://www.gutenberg.net

This Web site includes information about Project Gutenberg-tm,

including how to make donations to the Project Gutenberg Literary

Archive Foundation, how to help produce our new eBooks, and how to

subscribe to our email newsletter to hear about new eBooks.