DSP Benchmark documentation

Table of contents

Script documentation	2

Applications needed	2

Constants’ configuration	2

Program’s parameters	2

Path configuration	3

Program locations	3

Launching the script	3

The C program pattern	4

The Scilab data comparison script	4

The output file	5

Why is it not working?	5

By default, the file “benchmark” is located in the directory “[ DSPLIB ]/UTILS\SCRIPTS\BENCHMARK” and is set to perform a benchmark on the application located in the directory “[ DSPLIB ]/EXAMPLES/BENCHMARK/”. To try it, open a shell (cygwin) and launch the script “./benchmark”. You might have to specify some program paths or directory locations and install specified programs. The script will inform you about requirements.

# Script documentation

This script permits to workbench a program, without any intervention. Once it is well configured, it will automatically compile the program to evaluate, download it into the target, retrieves the data and computes them to extract the error information. It will store into an output file the cycle count this process took, the duration in seconds, the error average and the maximal error of the results.

To make this script works you will need a serial cable that you have to connect to the port COM1 of you PC. The other end of this cable has to be connected on the USART of the target.

To configure this script, edit the file “benchmark” and modify the values of the variables defines in the section “BENCH CONFIG”.

## Applications needed

cygwin, DataGet, DataExtract, scilab, bc, gawk, expr, echo, cp, mv, rm, make, sed, sort, uniq, printf, sh, which, rxvt.

## Constants’ configuration

This is the definition of the constants used in the header file generated by this script in order to configure the C program to workbench.

param1_def, param2_def, param3_def, param4_def

Those variables contain the names of the constants.

i.e.: 'VECT2_SIZE'

param1, param2, param3, param4

Those variables are defined by a list containing the values of the actual parameters (param1_def, param2_def, param3_def and param4_def).

The values of the list are separated by the character ‘#’. If the value has to be a blank, use the special string ‘SPACE’ to specify this case.

/!\ All the parameters have to be used.

i.e.: '32#64#128#256'

## Program’s parameters

This is the definition of the parameters that permit computations according to the actual configuration of the C program.

FREQ

This variable defines the main clock frequency of the target. It is used in the time calculation that took the program in its computation. This variable can be dependant of the parameters.

i.e.: If the clock frequency of the target depends on the parameter number 4, you can define it as follow: FREQ='$cur_param4*1000000'.

QA, QB

This is the Q formatted parameters that define the actual format to use. The resulting format will be a “Q QA . QB” format.

i.e.: QA='8' and QB='24' that define a 32-bit Q8.24 format number.

OUTPUT_DATA_TYPE

The output data type of the resulting vector that returns the target. Its value must be ‘real’ or ‘imag’.

i.e.: 'real'

## Path configuration

Important path used in the script.

CUR_PATH

Specify the directory where is located the Makefile you are using to compile the program (from the script path).

i.e.: '/cygdrive/d/trunk/SERVICES/DSPLIB/EXAMPLES/BENCH/AT32UC3A/GCC'

CONFIG_FILE_PATH

Specify the path of the output header file used to parameter the C program to workbench (from the $CUR_PATH directory).

i.e.: '../../bench_config.h'

SCRIPT_ERROR_FILE_PATH

Specify the path of the Scilab script used to generate the reference output signal of the C program in order to calculate the error (from the $CUR_PATH directory).

i.e.: "`pwd`/RESSOURCES/ref_conv.sce"

## Program locations

Specify the programs’ path from the $CUR_PATH directory.

DATAGET

Specify the path of the DataGet application.

i.e.: "`pwd`/../../SOURCES/WINDOWS/DATA_GET/DataGet.exe"

DATAEXTRACT

Specify the path of the DataExtract application.

i.e.:"`pwd`/../../SOURCES/WINDOWS/DATA_EXTRACT/DataExtract.exe"

SCILAB_PATH

Specify the path of the Scilab directory.

i.e.: '/cygdrive/c/Program Files/scilab-4.1'

## Launching the script

To run the script, go to the directory where is located the script and type: “./benchmark”.

By default the script is set to be performed on a the “[ DSPLIB ]/EXAMPLES/BENCHMARK/” example. To change the path, refer to the “path configuration” section.

The script will tell you which application is missing to perform the script.

# The C program pattern

In order to make the script compatible with the C program, you should use this C program pattern to design your benchmark. This pattern works perfectly with the EVK1100 board.

#include <avr32/io.h>

#include "compiler.h"

#include "board.h"

#include "dsp.h"

#include "pm.h"

#include "count.h"

#include "bench_config.h"

//! The main function

int main(int argc, char *argv[])

{

unsigned int cycle_count;

// Swith to external Oscillator 0.

pm_switch_to_osc0(&AVR32_PM, FOSC0, OSC0_STARTUP);

// Initialize the DSP debug module

dsp_debug_initialization(FOSC0);

// Get the actual cycle count

cycle_count = Get_sys_count();

// <<< Type here the code you need to workbench >>>

// Calculate the number of cycles

cycle_count = Get_sys_count() - cycle_count;

// Print the number of cycles

dsp16_debug_printf("Cycle count: %d\n", cycle_count);

// Print the output

// <<< use the functions dsp16_debug_print_vect, dsp32_debug_print_vect, dsp16_debug_print_complex_vect or dsp32_debug_print_complex_vect >>>

while(1);

}

In this program you will have access to the constants you defined in the script file as parameters. Those constants are accessible by including the file “bench_config.h”.

Finally the program has to be compiled defining DSP_BENCH=1 and DSP_DEBUG=1.

# The Scilab data comparison script

This is the file that contains the reference signal which should be the result of the output of the program to workbench. It serves to compare this reference signal with the actual output and permits to retrieve the error average and the maximal error of this output signal.

In this file you will just have to generate the signal you should receive with you C program. To do that you will have access to the constants you defined in the script file as parameters. The output signal must be store in the “y” variable and can be a real or a imaginary vector.

Here is an example for the FFT:

y_size = 2^NLOG;

t = 0:(y_size-1);

vect2_input = sin(2*%pi*4000*t/40000);

y = fft(vect2_input)/y_size;

# The output file

The resulting file of this benchmark is a text file containing all the workbench results. Each result is printed on 4 lines: the first line corresponds to the cycle count of the process, the next one is the duration of this process in second, the third is the error average of the result and the last, its maximal error. Then you can easily manage such a file using a spreadsheet.

# Why is it not working?

- The HyperTerminal is opened and so the application DataGet cannot open the COM port.

  - Close the HyperTerminal and rerun the script.

- An error is generated by the Scilab data comparison script.

  - Check the error on the file “dsp_bench_temp” else try to launch this script manually with the required files.

- An error occurred while compiling the program.

  - This might be due to an error on your C files. Compile it separately to check the error or look at the file ./dsp_bench_log.txt.