DSP Library

Requirements – version 1.0

1.	Description	4

1.1.	Types	4

1.2.	Function’s naming convention	4

1.3.	Compilation	4

1.3.1.	Compatibility	4

1.3.2.	Options	4

2.	DSP Library	5

2.1.	* Operators	5

2.1.1.	* Cosine	5

2.1.2.	* Sine	5

2.1.3.	* Arc Cosine	5

2.1.4.	* Arc Sine	5

2.1.5.	* Square root	5

2.1.6.	* Power	5

2.1.7.	* Exponential	5

2.1.8.	* Natural logarithm	5

2.2.	Vectors	5

2.2.1.	Addition	5

2.2.2.	Subtraction	5

2.2.3.	Multiplication	6

2.2.4.	Division	6

2.2.5.	Addition real	6

2.2.6.	Subtraction real	6

2.2.7.	Multiplication real	6

2.2.8.	Division real	6

2.2.9.	Dot Multiplication	6

2.2.10.	Dot Division	7

2.2.11.	Minimum	7

2.2.12.	Maximum	7

2.2.13.	Normalize	7

2.2.14.	Zero Padding	7

2.2.15.	Copy	7

2.2.16.	Negate	7

2.2.17.	Dot Power	8

2.2.18.	Convolution	8

2.2.19.	Cross Correlation	8

2.3.	Transforms	8

2.3.1.	Fast Fourier Transform (FFT)	8

2.3.2.	Complex Fast Fourier Transform (Complex FFT)	8

2.3.3.	Inverse Fast Fourier Transform (IFFT)	9

2.3.4.	Discrete Cosine Transform (DCT)	9

2.3.5.	Inverse Discrete Cosine Transform (IDCT)	9

2.4.	Filtering	9

2.4.1.	Finite Impulse Response (FIR)	9

2.4.2.	Infinite Impulse Response (IIR)	10

2.4.3.	* Butterworth	10

2.4.4.	* Bessel, Tchebychev, Lagrange, …	10

2.4.5.	* Adaptive filtering	10

2.4.5.1.	* Least Mean-Square (LMS)	10

2.4.5.2.	* Kalman	10

2.4.5.3.	* Wiener	10

2.5.	Windowing	10

2.5.1.	Rectangular	10

2.5.2.	Triangular	10

2.5.3.	Blackman	11

2.5.4.	Hamming	11

2.5.5.	Gauss	11

2.5.6.	Hanning	11

2.5.7.	Kaiser	11

2.5.8.	Bartlett	12

2.6.	Signal generation	12

2.6.1.	Periodic	12

2.6.1.1.	Sine	12

2.6.1.2.	Cosine	12

2.6.1.3.	Rectangle	12

2.6.1.4.	Square	13

2.6.1.5.	Triangular	13

2.6.1.6.	Dirac comb	13

2.6.2.	Non-periodic	13

2.6.2.1.	Ramp	13

2.6.2.2.	Echelon	13

2.6.2.3.	Dirac	14

2.6.3.	Noise	14

2.6.3.1.	Noise	14

2.6.3.2.	White noise	14

2.6.3.3.	Pink noise	14

3.	DSP Applications Library	15

3.1.	Telephony	15

3.1.1.	Echo cancellation	15

3.1.2.	Calling Number Identification (CNID)	15

3.1.3.	Automatic Gain Control (AGC)	15

3.1.4.	Dual Tone Multi Frequency detection (DTMF)	15

3.2.	Soft modem	15

3.2.1.	V90 – V91	15

3.3.	Compression	15

3.3.1.	Adaptive Differential Pulse Code Modulation (ADPCM)	15

3.4.	Regulator	15

3.4.1.	Proportional Integral Derivative (PID)	15

(*) Those functions will be implemented only if I need to.

# Description

All the following functions will be coded in C language. To improve their performance, assembly language will be also used. A specific define will be used to allow the assembly language and so make those functions more efficient but not generic anymore.

## Types

Each function is designed to work with 16 and 32 bits signed values. The 16 bits type is used when you favor speed instead of accuracy else you can use the 32 bits type.

All types include the fixed point notion which values allowed are in the range [-1; +1].

The types must be of three different kinds: basic types, complex types and vector types.

## Function’s naming convention

A function must follow this naming convention: aX_y_z(…)

Where:

a is the name of the library (dsp or dspapp)

X is the type used (16 or 32)

y is category of the library (op, vect, trans, …)

z is the name of the function (cos, mul, …)

Each function name must have a maximal length of 32 characters.

Example:

dsp16_vect_mul is a function part of the dsp library. It works with 16 bits values and permits to multiply vectors.

## Compilation

### Compatibility

The whole Library has to be “IAR” and “GCC” compatible.

The generic C functions are written in C ANSI languages and can be compiled for any other platform.

### Options

When you compile the library, you can specify different options that permit to optimize the algorithmic according to your needs. One option will minimize the size of the algorithms and their resources and the other option will increase the precision of the data computed by the algorithms. By default, if no option is specified, the algorithms will be optimized to run as fast as they can.

# DSP Library

## * Operators

### * Cosine

- Name: dspXX_op_cos

### * Sine

- Name: dspXX_op_sin

### * Arc Cosine

- Name: dspXX_op_acos

### * Arc Sine

- Name: dspXX_op_asin

### * Square root

- Name: dspXX_op_sqrt

### * Power

- Name: dspXX_op_pow

### * Exponential

- Name: dspXX_op_exp

### * Natural logarithm

- Name: dspXX_op_ln

## Vectors

### Addition

- Name: dspXX_vect_add

- Use: dspXX_vect_add(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 + vect3

- Description: Add 2 vectors (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 += vect3).

### Subtraction

- Name: dspXX_vect_sub

- Use: dspXX_vect_sub(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 - vect3

- Description: Subtract 2 vectors (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 -= vect3).

### Multiplication

- Name: dspXX_vect_mul

- Use: dspXX_vect_mul(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 * vect3

- Description: Multily 2 vectors (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 *= vect3).

### Division

- Name: dspXX_vect_div

- Use: dspXX_vect_div(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 / vect3

- Description: Divide 2 vectors (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 /= vect3).

### Addition real

- Name: dspXX_vect_addr

- Use: dspXX_vect_addr(vect1, vect2, n, size)

- Operation: vect1 = vect2 + n

- Description: Add 1 vector (vect2) with a real (n) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 += n).

### Subtraction real

- Name: dspXX_vect_subr

- Use: dspXX_vect_subr(vect1, vect2, n, size)

- Operation: vect1 = vect2 - n

- Description: Subtract 1 vector (vect2) with a real (n) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 -= n).

### Multiplication real

- Name: dspXX_vect_mulr

- Use: dspXX_vect_mulr(vect1, vect2, n, size)

- Operation: vect1 = vect2 * n

- Description: Multily 1 vector (vect2) with a real (n) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 *= n).

### Division real

- Name: dspXX_vect_divr

- Use: dspXX_vect_divr(vect1, vect2, n, size)

- Operation: vect1 = vect2 / n

- Description: Divide 2 1 vector (vect2) with a real (n) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 /= n).

### Dot Multiplication

- Name: dspXX_vect_dotmul

- Use: dspXX_vect_dotmul(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 .* vect3

- Description: Multiply 2 vectors element by element (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 .*= vect3).

### Dot Division

- Name: dspXX_vect_dotdiv

- Use: dspXX_vect_dotdiv(vect1, vect2, vect3, size)

- Operation: vect1 = vect2 ./ vect3

- Description: Divide 2 vectors element by element (vect2 and vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 ./= vect3).

### Minimum

- Name: dspXX_vect_min

- Use: dspXX_vect_min(vect1, size)

- Operation: min = MIN(vect2)

- Description: Return the minimum of a vector (vect1).

### Maximum

- Name: dspXX_vect_max

- Use: dspXX_vect_max(vect1, size)

- Operation: max = MAX(vect2)

- Description: Return the maximum of a vector (vect1).

### Normalize

- Name: dspXX_vect_norm

- Use: dspXX_vect_norm(vect1, vect2, size)

- Operation: vect1 = norm(vect2)

- Description: Normalize a vector (vect2) and store the result in another vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 = norm(vect1)).

### Zero Padding

- Name: dspXX_vect_zeropad

- Use: dspXX_vect_zeropad(vect1, size, num_zero)

- Description: Zero pads the tail of the vector (vect1).

### Copy

- Name: dspXX_vect_copy

- Use: dspXX_vect_copy(vect1, vect2, size)

- Operation: vect1 = vect2

- Description: Copy one vector (vect2) into the other (vect1).

### Negate

- Name: dspXX_vect_neg

- Use: dspXX_vect_neg(vect1, vect2)

- Operation: vect1 = -vect2

- Description: Negate a vector (vect2) and store the result in a 2nd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 = -vect1).

### Dot Power

- Name: dspXX_vect_pow

- Use: dspXX_vect_pow(vect1, vect2, n)

- Operation: vect1 = vect2.^n

- Description: Raise the element of a vector (vect2) to the power of a numerical value (n) and store the result in a 2nd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 = vect1.^n).

- Requirement: pow

### Convolution

- Name: dspXX_vect_conv

- Alias name: conv

- Use: dspXX_vect_conv(vect1, vect2, vect3, size)

- Operation: vect1 = conv(vect2, vect3)

- Description: Convolute a vector (vect2) with another (vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 = conv(vect1, vect3)).

- Applications: filters, signal frequency motion.

### Cross Correlation

- Name: dspXX_vect_corr

- Alias name: xcorr

- Use: dspXX_vect_corr (vect1, vect2, vect3, size)

- Operation: vect1 = xcorr(vect2, vect3)

- Description: Correlate a vector (vect2) with another (vect3) and store the result in a 3rd vector (vect1). If vect1 is equal to vect2, this function should be optimized (vect1 = xcorr(vect1, vect3)).

- Applications: signal comparison.

## Transforms

### Fast Fourier Transform (FFT)

- Name: dspXX_trans_fft

- Alias name: fft

- Use: dspXX_trans_fft(vect1, vect2, size, n)

- Operation: vect1 = fft(vect2)

- Description: Returns the 2^n-point discrete Fourier transform (DFT) of vector vect2, computed with a fast Fourier transform (FFT) algorithm. The result is stored into a complex vector (vect1).

- Applications: spectrums, frequency analyzing.

- Requirement: vectors

### Complex Fast Fourier Transform (Complex FFT)

- Name: dspXX_trans_complexfft

- Alias name: fft

- Use: dspXX_trans_complexfft(complex_vect1, vect2, size, n)

- Operation: complex_vect1 = complexfft(vect2)

- Description: Returns the 2^n-point discrete Fourier transform (DFT) of vector vect2, computed with a fast Fourier transform (FFT) algorithm. The result is stored into a complex vector (complex_vect1).

- Applications: spectrums, frequency analyzing.

- Requirement: vectors

### Inverse Fast Fourier Transform (IFFT)

- Name: dspXX_trans_ifft

- Alias name: ifft

- Use: dspXX_trans_ifft(complex_vect1, complex_vect2, size, n)

- Operation: complex_vect1 = ifft(complex_vect2)

- Description: Returns the original signal issued from an n-point discrete Fourier transform (DFT), computed with a fast Fourier transform (FFT) algorithm (complex_vect2). The result is stored into another complex vector (complex_vect1).

- Requirement: vectors

### Discrete Cosine Transform (DCT)

- Name: dspXX_trans_dct

- Alias name: dct

- Use: dspXX_trans_dct(vect1, vect2, size)

- Operation: vect1 = dct(vect2)

- Description: Compute a Discrete Cosine Transform of a vector (vect2) and the result in another vector (vect1).

- Applications: lossy data compression.

- Requirement: cos, vectors

### Inverse Discrete Cosine Transform (IDCT)

- Name: dspXX_trans_idct

- Alias name: idct

- Use: dspXX_trans_idct(vect1, vect2, size)

- Operation: vect1 = idct(vect2)

- Description: Returns the original signal issued from a Discrete Cosine Transform (vect2). This result is stored in another vector (vect1).

- Applications: lossy data decompression.

- Requirement: acos, vectors

## Filtering

### Finite Impulse Response (FIR)

- Name: dspXX_filt_fir

- Alias name: filter

- Use: dspXX_filt_fir(vect1, vect2, vect_num, size, num_order)

- Operation: vect1 = fir(vect2, vect_num)

- Description:  Apply a filter (vect_num) to a signal (vect2) and store the result in another vector (vect1).

- Applications : filters

- Requirement: vectors

### Infinite Impulse Response (IIR)

- Name: dspXX_filt_iir

- Alias name: filter

- Use: dspXX_filt_iir(vect1, vect2, vect_num, vect_den, size, num_order, den_order)

- Operation: vect1 = iir(vect2, vect_num/vect_den)

- Description:  Apply a filter (vect_num and vect_den) to a signal (vect2) and store the result in another vector (vect1).

- Applications : filters

- Requirement: vectors

### * Butterworth

- Name: dspXX_filt_butterworth

- Use: dspXX_filt_butterworth(vect1, vect2, size, fc, order)

- Operation: vect1 = butterworth(vect2, fc, order)

- Description:  Apply a butterworth low-pass filter with a specified cut frequency (fc) and a specified order (order) to a signal (vect2) and store the result in another vector (vect1).

- Applications : low-pass filters

- Requirement: vectors, conv

### * Bessel, Tchebychev, Lagrange, …

### * Adaptive filtering

#### * Least Mean-Square (LMS)

#### * Kalman

#### * Wiener

## Windowing

### Rectangular

- Name: dspXX_win_rect

- Use: dspXX_win_rect(vect1, vect2, size, f, width)

- Operation: vect1 = win_rectangle(vect2, f, width)

- Description:  Window a signal (vect2) using a rectangle windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Triangular

- Name: dspXX_win_tri

- Use: dspXX_win_tri(vect1, vect2, size, f, width)

- Operation: vect1 = win_triangular(vect2, f, width)

- Description:  Window a signal (vect2) using a triangular windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Blackman

- Name: dspXX_win_black

- Use: dspXX_win_black(vect1, vect2, size, f, width)

- Operation: vect1 = win_blackman(vect2, f, width)

- Description:  Window a signal (vect2) using a blackman windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Hamming

- Name: dspXX_win_hamm

- Use: dspXX_win_hamm(vect1, vect2, size, f, width)

- Operation: vect1 = win_hamming(vect2, f, width)

- Description:  Window a signal (vect2) using a hamming windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Gauss

- Name: dspXX_win_gauss

- Use: dspXX_win_gauss(vect1, vect2, size, f, width)

- Operation: vect1 = win_gauss(vect2, f, width)

- Description:  Window a signal (vect2) using a gauss windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Hanning

- Name: dspXX_win_hann

- Use: dspXX_win_hann(vect1, vect2, size, f, width)

- Operation: vect1 = win_hanning(vect2, f, width)

- Description:  Window a signal (vect2) using a hanning windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Kaiser

- Name: dspXX_win_kaiser

- Use: dspXX_win_kaiser(vect1, vect2, size, f, width)

- Operation: vect1 = win_kaiser(vect2, f, width)

- Description:  Window a signal (vect2) using a kaiser windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

### Bartlett

- Name: dspXX_win_bart

- Use: dspXX_win_bart(vect1, vect2, size, f, width)

- Operation: vect1 = win_bartlett(vect2, f, width)

- Description:  Window a signal (vect2) using a bartlett windowing centered on a specified frequency (f) and with a specified width (width). The signal resulting is stored in another vector (vect1).

- Applications : windowing

- Requirement: fft, ifft, vectors

## Signal generation

### Periodic

#### Sinusoidal

- Name: dspXX_gen_sin

- Use: dspXX_gen_sin(vect1, size, f, fs, phase)

- Operation: vect1 = sin(2*pi*f/fs*t + phase)

- Description:  Generate a sinusoidal signal with a specified frequency (f), sampling frequency (fs) and phase (phase). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: operators, vectors

#### Cosinusoidal

- Name: dspXX_gen_cos

- Use: dspXX_gen_cos(vect1, size, f, fs, phase)

- Operation: vect1 = cos(2*pi*f/fs*t + phase)

- Description:  Generate a cosinusoidal signal with a specified frequency (f), sampling frequency (fs) and phase (phase). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: operators, vectors

#### Rectangle

- Name: dspXX_gen_rect

- Use: dspXX_gen_rect(vect1, size, f, fs, width, delay)

- Operation: vect1 = rect(f, fs, width, delay)

- Description:  Generate a rectangular signal with a specified frequency (f), sampling frequency (fs), pulse width (width) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Square

- Name: dspXX_gen_sqr

- Use: dspXX_gen_sqr(vect1, size, f, fs, delay)

- Operation: vect1 = square(f, fs, delay)

- Description:  Generate a square signal with a specified frequency (f), sampling frequency (fs) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Triangular

- Name: dspXX_gen_trg

- Use: dspXX_gen_trg(vect1, size, f, fs, delay)

- Operation: vect1 = triangle(f, fs, delay)

- Description:  Generate a triangular signal with a specified frequency (f), sampling frequency (fs) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Dirac comb

- Name: dspXX_gen_dcomb

- Use: dspXX_gen_dcomb(vect1, size, f, fs, delay)

- Operation: vect1 = dirac_comb(f, fs, delay)

- Description:  Generate a Dirac comb signal with a specified frequency (f), sampling frequency (fs) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

### Non-periodic

#### Ramp

- Name: dspXX_gen_ramp

- Use: dspXX_gen_ramp(vect1, size, fs, incline, delay)

- Operation: vect1 = ramp(f, fs, incline, delay)

- Description:  Generate a ramp signal with a sampling frequency (fs), inclination (incline) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Echelon

- Name: dspXX_gen_e

- Use: dspXX_gen_e(vect1, size, fs, delay)

- Operation: vect1 = echelon(f, fs, delay)

- Description:  Generate an echelon signal with a sampling frequency (fs) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Dirac

- Name: dspXX_gen_d

- Use: dspXX_gen_d(vect1, size, fs, delay)

- Operation: vect1 = dirac(f, fs, delay)

- Description:  Generate a Dirac signal with a sampling frequency (fs) and time delay (delay). The signal resulting is stored in another vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

### Noise

#### Noise

- Name: dspXX_gen_noise

- Use: dspXX_gen_noise(vect1, size, amp)

- Operation: vect1 = amp*rand()

- Description:  Generate a noise with specified amplitude (amp). The signal resulting is stored in a vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### White noise

- Name: dspXX_gen_wnoise

- Use: dspXX_gen_wnoise(vect1, size, amp)

- Operation: vect1 = white_noise();

- Description:  Generate a white noise with specified amplitude (amp). The signal resulting is stored in a vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

#### Pink noise

- Name: dspXX_gen_pnoise

- Use: dspXX_gen_pnoise(vect1, size, amp)

- Operation: vect1 = pink_noise();

- Description:  Generate a pink noise with specified amplitude (amp). The signal resulting is stored in a vector (vect1).

- Applications: modulation, demodulation…

- Requirement: vectors

# DSP Applications Library

## Telephony

### Echo cancellation

### Calling Number Identification (CNID)

### Automatic Gain Control (AGC)

### Dual Tone Multi Frequency detection (DTMF)

## Soft modem

### V90 – V91

## Compression

### Adaptive Differential Pulse Code Modulation (ADPCM)

## Regulator

### Proportional Integral Derivative (PID)