# Annexes

| Table 1.1.1 - Benchmark of 16-bit Radix-4 D.I.T. complex FFT: generic |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimization | T.F.T.* in SRAM (cycles) |  | T.F.T.* in FLASH (cycles) |  | Error |  | Algorithm’s size in memory (bytes) (code size + T.F.T.* size) |
|  |  |  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |  |
|  |  | wait-state |  | wait-state |  |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |  |
| 64-point | (0) | 6,296 (209.9us) | 6,489 (108.2us) | 6,640 (221.3us) | 7,012 (116.9us) | 2.98 | 15.63 | 1K (840 + 194) |
|  | (1) | 6,343 (211.4us) | 6,538 (109.0us) | 6,777 (225.9us) | 7,167 (119.5us) | 1.58 | 6.53 | 1K (844 + 194) |
|  | (2) | 6,666 (222.2us) | 6,959 (116.0us) | 7,092 (236.4us) | 7,546 (125.8us) | 2.98 | 15.63 | 1.1K (1032 + 66) |
|  | (3) | 6,747 (224.9us) | 6,996 (116.6us) | 7,101 (236.7us) | 7,536 (125.6us) | 1.58 | 6.53 | 1K (984 + 66) |
| 256-point | (0) | 33,723 (1124.1us) | 34,682 (578.0us) | 35,265 (1175.5us) | 37,033 (617.2us) | 3.02 | 21.92 | 1.6K (840 + 770) |
|  | (1) | 34,041 (1134.7us) | 35,003 (583.4us) | 35,988 (1199.6us) | 37,836 (630.6us) | 1.69 | 8.80 | 1.6K (844 + 770) |
|  | (2) | 35,304 (1176.8us) | 36,675 (611.3us) | 37,194 (1239.8us) | 39,294 (654.9us) | 3.02 | 21.92 | 1.3K (1032 + 258) |
|  | (3) | 35,,826 (1194.2us) | 37,032 (617.2us) | 37,419 (1247.3us) | 39,453 (657.6us) | 1.69 | 8.80 | 1.2K (984 + 258) |
| 1024-point | (0) | 169,006 (5.63ms) | 173,611 (2.90ms) | 175,394 (5.85ms) | 183,358 (3.06ms) | 3.04 | 25.01 | 3.8K (840 + 3K) |
|  | (1) | 170,795 (5.69ms) | 175,404 (2.92ms) | 178,863 (5.96ms) | 187,161 (3.12ms) | 1.67 | 12.31 | 3.8K (844 + 3K) |
|  | (2) | 175,446 (5.85ms) | 181,703 (3.03ms) | 183,248 (6.11ms) | 192,530 (3.21ms) | 3.04 | 25.01 | 2K (1032 + 1K) |
|  | (3) | 178,153 (5.94ms) | 183,772 (3.06ms) | 184,761 (6.16ms) | 193,802 (3.23ms) | 1.67 | 12.31 | 2K (984 + 1K) |
| 4096-point | (0) | 812,321 (27.08ms) | 833,820 (13.90ms) | 838,147 (27.94ms) | 873,235 (14.55ms) | 3.09 | 32.90 | 12.8K (840 + 12K) |
|  | (1) | 821,533 (27.38ms) | 843,037 (14.05ms) | 854,154 (28.47ms) | 890,598 (14.84ms) | 1.52 | 14.60 | 12.8K (844 + 12K) |
|  | (2) | 838,212 (27.94ms) | 866,315 (14.44ms) | 869,718 (28.99ms) | 910,054 (15.17ms) | 3.09 | 32.90 | 5K (1032 + 4K) |
|  | (3) | 851,232 (28.37ms) | 876,816 (14.61ms) | 877,959 (29.27ms) | 917,367 (15.29ms) | 1.52 | 14.60 | 5K (984 + 4K) |

*: Twiddle Factors Table.

(0): Algorithmic optimized for speed.

(1): Algorithmic optimized for accuracy.

(2): Algorithmic optimized for size.

(3): Algorithmic optimized for size and accuracy.

| Table 1.1.2 - Benchmark of 16-bit Radix-4 D.I.T. complex FFT: avr32-uc3 optimized |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimization | T.F.T.* in SRAM (cycles) |  | T.F.T.* in FLASH (cycles) |  | Error |  | Algorithm’s size in memory (bytes) (code size + T.F.T.* size) |
|  |  |  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |  |
|  |  | wait-state |  | wait-state |  |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |  |
| 64-point | (0) | 2,611 (87.0us) | 2,661 (44.4us) | 2,753 (91.8us) | 2,877 (48.0us) | 3.00 | 10.04 | 894 (700 + 194) |
|  | (1) | 2,951 (98.4us) | 2,999 (50.0us) | 3,097 (103.2us) | 3,265 (54.4us) | 1.63 | 6.53 | 774 (580 + 194) |
|  | (2) | 2,833 (94.4us) | 2,912 (48.5us) | 3,027 (100.9us) | 3,221 (53.7us) | 2.93 | 10.04 | 710 (644 + 66) |
|  | (3) | 3,206 (106.9us) | 3,311 (55.2us) | 3,458 (115.3us) | 3,683 (61.4us) | 1.63 | 6.53 | 766 (700 + 66) |
| 256-point | (0) | 13,661 (455.4us) | 13,932 (232.2us) | 14,306 (476.9us) | 14,904 (248.4us) | 2.78 | 13.86 | 1.4K (700 + 770) |
|  | (1) | 15,777 (525.9us) | 16,033 (267.2us) | 16,428 (547.6us) | 17,242 (287.4us) | 1.68 | 7.46 | 1.3K (580 + 770) |
|  | (2) | 14,651 (488.4us) | 15,056 (250.9us) | 15,527 (517.6us) | 16,442 (274.0us) | 2.87 | 15.82 | 902 (644 + 258) |
|  | (3) | 16,916 (563.9us) | 17,469 (291.2us) | 18,041 (601.4us) | 19,152 (319.2us) | 1.68 | 7.46 | 958 (700 + 258) |
| 1024-point | (0) | 67,671 (2.26ms) | 69,027 (1.15ms) | 70,355 (2.35ms) | 73,059 (1.22ms) | 2.84 | 19.33 | 3.7K (700 + 3K) |
|  | (1) | 79,195 (2.64ms) | 80,475 (1.34ms) | 81,887 (2.73ms) | 85,507 (1.43ms) | 1.69 | 10.23 | 3.6K (580 + 3K) |
|  | (2) | 71,765 (2.39ms) | 73,680 (1.23ms) | 75,403 (2.51ms) | 79,423 (1.32ms) | 2.86 | 19.33 | 1.6K (644 + 1K) |
|  | (3) | 83,906 (2.80ms) | 86,635 (1.44ms) | 88,560 (2.95ms) | 93,629 (1.56ms) | 1.69 | 10.23 | 1.7K (700 + 1K) |
| 4096-point | (0) | 322,897 (10.76ms) | 329,370 (5.49ms) | 333,764 (11.13ms) | 345,678 (5.76ms) | 2.80 | 23.69 | 12.7K (700 + 12K) |
|  | (1) | 381,269 (12.71ms) | 387,413 (6.46ms) | 392,146 (13.07ms) | 407,788 (6.80ms) | 1.58 | 11.84 | 12.6K (580 + 12K) |
|  | (2) | 339,439 (11.31ms) | 348,176 (5.80ms) | 354,159 (11.81ms) | 371,396 (6.19ms) | 2.81 | 23.69 | 4.6K (644 + 4K) |
|  | (3) | 400,304 (13.34ms) | 413,273 (6.89ms) | 419,111 (13.97ms) | 441,578 (7.36ms) | 1.58 | 11.84 | 4.7K (700 + 4K) |

*: Twiddle Factors Table.

(0): Algorithmic optimized for speed.

(1): Algorithmic optimized for accuracy.

(2): Algorithmic optimized for size.

(3): Algorithmic optimized for size and accuracy.

| Table 1.2.1 - Benchmark of 32-bit Radix-4 D.I.T. complex FFT: generic |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimization | T.F.T.* in SRAM (cycles) |  | T.F.T.* in FLASH (cycles) |  | Error |  | Algorithm’s size in memory (bytes) (code size + T.F.T.* size) |
|  |  |  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |  |
|  |  | wait-state |  | wait-state |  |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |  |
| 64-point | (0) | 13,206 (440.2us) | 13,509 (225.2us) | 13,580 (452.7us) | 14,063 (234.4us) | 0.70 | 5.70 | 2.2K (1816 + 392) |
|  | (1) | 15,323 (510.8us) | 15,659 (261.0us) | 15,627 (520.9us) | 16,113 (268.6us) | 0.60 | 5.70 | 2.4K (2112 + 392) |
|  | (2) | 13,622 (454.1us) | 14,011 (233.5us) | 13,938 (464.6us) | 14,497 (241.6us) | 0.70 | 5.70 | 2.0K (1952 + 136) |
|  | (3) | 15,714 (523.8us) | 16,245 (270.8us) | 16,058 (535.3us) | 16,805 (280.1us) | 0.60 | 5.70 | 2.3K (2248 + 136) |
| 256-point | (0) | 74,297 (2.48ms) | 75,940 (1.27ms) | 75,992 (2.53ms) | 78,445 (1.31ms) | 0.50 | 4.80 | 3.3K (1816 + 1.5K) |
|  | (1) | 87,791 (2.93ms) | 89,605 (1.49ms) | 89,165 (2.97ms) | 91,636 (1.53ms) | 0.30 | 4.80 | 3.6K (2112 + 1.5K) |
|  | (2) | 76,136 (2.54ms) | 78,160 (1.30ms) | 77,537 (2.58ms) | 80,329 (1.34ms) | 0.50 | 4.80 | 2.4K (1952 + 520) |
|  | (3) | 89,543 (2.98ms) | 92,446 (1.54ms) | 91,058 (3.04ms) | 94,978 (1.59ms) | 0.30 | 4.80 | 2.7K (2248 + 520) |
| 1024-point | (0) | 383,212 (12.77ms) | 391,715 (6.53ms) | 390,260 (13.01ms) | 402,123 (6.70ms) | 0.50 | 8.80 | 7.8K (1816 + 6K) |
|  | (1) | 457,571 (15.25ms) | 466,815 (7.78ms) | 463,279 (15.44ms) | 475,223 (7.92ms) | 0.30 | 6.10 | 8.1K (2112 + 6K) |
|  | (2) | 390,794 (13.03ms) | 400,869 (6.68ms) | 396,576 (13.22ms) | 409,841 (6.83ms) | 0.50 | 8.80 | 3.9K (1952 + 2K) |
|  | (3) | 464,988 (15.50ms) | 479,847 (8.00ms) | 471,226 (15.71ms) | 490,367 (8.17ms) | 0.30 | 6.10 | 4.2K (2248 + 2K) |

*: Twiddle Factors Table.

(0): Algorithmic optimized for speed.

(1): Algorithmic optimized for accuracy.

(2): Algorithmic optimized for size.

(3): Algorithmic optimized for size and accuracy.

| Table 2.1.1 - Benchmark of 16-bit Convolution: generic |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1st input signal | 2nd input signal | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 32-point | 32-points | 15,681 (522.7us) | 16,344 (272.4us) | 1.60 | 3.90 |
|  | 64-points | 23,524 (784.1us) | 24,508 (408.5us) | 2.00 | 4.50 |
|  | 128-points | 39,204 (1.31ms) | 40,830 (680.5us) | 1.90 | 4.50 |
|  | 256-points | 70,564 (2.35ms) | 73,470 (1.22ms) | 1.70 | 4.10 |
| 64-point | 64-points | 57,757 (1.93ms) | 60,084 (1.00ms) | 1.80 | 4.70 |
|  | 128-points | 86,752 (2.89ms) | 90,234 (1.50ms) | 1.80 | 4.40 |
|  | 256-points | 144,736 (4.82ms) | 150,522 (2.51ms) | 1.50 | 4.80 |
| 128-point | 128-points | 221,780 (7.39ms) | 230,512 (3.84ms) | 1.80 | 5.30 |
|  | 256-points | 333,018 (11.10ms) | 346,097 (5.77ms) | 1.70 | 5.00 |
| 256-point | 256-points | 869,316 (28.98ms) | 903,136 (15.05ms) | 1.70 | 5.40 |

| Table 2.1.2 - Benchmark of 16-bit Convolution: avr32-uc3 optimized |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1st input signal | 2nd input signal | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 32-point | 32-points | 5,571 (185.7us) | 6,127 (102.1us) | 1.60 | 3.90 |
|  | 64-points | 8,248 (274.9us) | 9,070 (151.2us) | 2.00 | 4.50 |
|  | 128-points | 13,592 (453.1us) | 14,944 (249.1us) | 1.90 | 4.50 |
|  | 256-points | 24,280 (809.3us) | 26,688 (444.8us) | 1.70 | 4.10 |
| 64-point | 64-points | 19,087 (636.2us) | 20,947 (349.1us) | 1.80 | 4.70 |
|  | 128-points | 28,532 (951.1us) | 31,308 (521.8us) | 1.80 | 4.40 |
|  | 256-points | 47,412 (1.58ms) | 52,012 (866.9us) | 1.50 | 4.80 |
| 128-point | 128-points | 70,694 (2.36ms) | 77,471 (1.29ms) | 1.80 | 5.30 |
|  | 256-points | 105,966 (3.53ms) | 116,099 (1.93ms) | 1.70 | 5.00 |
| 256-point | 256-points | 272,214 (9.07ms) | 298,031 (4.97ms) | 1.70 | 5.40 |

| Table 2.2.1 - Benchmark of 32-bit Convolution: generic |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1st input signal | 2nd input signal | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 32-point | 32-points | 28,359 (945.3us) | 29,182 (486.4us) | 0.40 | 1.60 |
|  | 64-points | 42,572 (1.42ms) | 43,788 (729.8us) | 0.40 | 2.10 |
|  | 128-points | 70,988 (2.37ms) | 72,990 (1.22ms) | 0.30 | 1.90 |
|  | 256-points | 127,820 (4.26ms) | 131,390 (2.19ms) | 0.40 | 1.70 |
| 64-point | 64-points | 109,179 (3.64ms) | 112,334 (1.87ms) | 0.40 | 1.70 |
|  | 128-points | 163,968 (5.47ms) | 168,678 (2.81ms) | 0.50 | 1.60 |
|  | 256-points | 273,536 (9.12ms) | 281,350 (4.69ms) | 0.60 | 2.70 |
| 128-point | 128-points | 429,026 (14.30ms) | 441,458 (7.36ms) | 0.40 | 2.30 |
|  | 256-points | 644,074 (21.47ms) | 662,677 (11.04ms) | 0.40 | 2.00 |
| 256-point | 256-points | 1,701,554 (56.72ms) | 1,750,962 (29.18ms) | 0.50 | 3.10 |

| Table 2.2.2 - Benchmark of 32-bit Convolution: avr32-uc3 optimized |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1st input signal | 2nd input signal | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 32-point | 32-points | 13,361 (445.4us) | 13,680 (228.0us) | 0.60 | 2.30 |
|  | 64-points | 19,958 (665.3us) | 20,414 (340.2us) | 0.50 | 2.10 |
|  | 128-points | 33,142 (1.10ms) | 33,872 (564.5us) | 0.50 | 1.90 |
|  | 256-points | 59,510 (1.98ms) | 60,784 (1.01ms) | 0.50 | 2.10 |
| 64-point | 64-points | 50,501 (1.68ms) | 51,624 (860.4us) | 0.50 | 1.70 |
|  | 128-points | 75,722 (2.52ms) | 77,376 (1.29ms) | 0.60 | 2.40 |
|  | 256-points | 126,154 (4.21ms) | 128,864 (2.15ms) | 0.70 | 2.70 |
| 128-point | 128-points | 196,972 (6.57ms) | 201,244 (3.35ms) | 0.50 | 2.30 |
|  | 256-points | 295,540 (9.85ms) | 301,887 (5.03ms) | 0.60 | 2.30 |
| 256-point | 256-points | 778,684 (25.96ms) | 795,388 (13.26ms) | 0.60 | 3.10 |

| Table 3.1.1 - Benchmark of 16-bit FIR Filter: generic |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of Taps | I.R.* in SRAM (cycles) |  | I.R.* in FLASH (cycles) |  | Error |  |
|  |  |  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |
| 64-point | 24 | 7,424 (247.5us) | 7,682 (128.0us) | 10,704 (356.8us) | 12,352 (205.9us) | 2.27 | 9.46 |
|  | 48 | 5,780 (192.7us) | 5,996 (99.9us) | 8,517 (283.9us) | 9,868 (164.5us) | 3.09 | 15.57 |
| 256-point | 24 | 41,793 (1.39ms) | 43,202 (720.0us) | 60,433 (2.01ms) | 69,760 (1.16ms) | 2.22 | 9.46 |
|  | 48 | 70,101 (2.34ms) | 72,620 (1.21ms) | 103,750 (3.46ms) | 120,268 (2.00ms) | 5.66 | 27.20 |
|  | 72 | 90,921 (3.03ms) | 94,262 (1.57ms) | 135,691 (4.52ms) | 157,528 (2.63ms) | 11.25 | 52.65 |
|  | 100 | 105,127 (3.50ms) | 108,903 (1.82ms) | 156,466 (5.22ms) | 185,989 (3.10ms) | 14.19 | 63.71 |
| 512-point | 24 | 87,617 (2.92ms) | 90,562 (1.51ms) | 126,737 (4.22ms) | 146,304 (2.44ms) | 2.23 | 9.46 |
|  | 48 | 155,861 (5.20ms) | 161,452 (2.69ms) | 230,726 (7.69ms) | 267,468 (4.46ms) | 5.78 | 27.20 |
|  | 72 | 216,617 (7.22ms) | 224,566 (3.74ms) | 323,339 (10.78ms) | 375,384 (6.26ms) | 10.79 | 52.65 |
|  | 100 | 276,391 (9.21ms) | 286,311 (4.77ms) | 411,442 (13.71ms) | 489,093 (8.15ms) | 14.05 | 63.71 |
| 1024-point | 24 | 179,265 (5.98ms) | 185,282 (3.09ms) | 259,345 (8.64ms) | 299,392 (4.99ms) | 2.21 | 9.46 |
|  | 48 | 327,381 (10.91ms) | 339,116 (5.65ms) | 484,678 (16.16ms) | 561,868 (9.36ms) | 5.85 | 27.20 |
|  | 72 | 468,009 (15.60ms) | 485,174 (8.09ms) | 698,635 (23.29ms) | 811,096 (13.52ms) | 10.69 | 52.65 |
|  | 100 | 618,919 (20.63ms) | 641,127 (10.69ms) | 921,394 (30.71ms) | 1,095,301 (18.26ms) | 13.99 | 63.71 |

*: Impulse Response.

| Table 3.1.2 - Benchmark of 16-bit FIR Filter: avr32-uc3 optimized |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of Taps | I.R.* in SRAM (cycles) |  | I.R.* in FLASH (cycles) |  | Error |  |
|  |  |  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |
| 64-point | 24 | 2,439 (81.3us) | 2,657 (44.3us) | 2,703 (90.1us) | 3,054 (50.9us) | 2.27 | 9.46 |
|  | 48 | 2,115 (70.5us) | 2,309 (38.5us) | 2,355 (78.5us) | 2,670 (44.5us) | 3.09 | 15.57 |
| 256-point | 24 | 12,712 (423.7us) | 13,841 (230.7us) | 14,128 (470.9us) | 15,966 (266.1us) | 2.22 | 9.46 |
|  | 48 | 21,604 (720.1us) | 23,573 (392.9us) | 24,148 (804.9us) | 27,390 (456.5us) | 5.66 | 27.20 |
|  | 72 | 28,192 (939.7us) | 30,785 (513.1us) | 31,576 (1.05ms) | 35,862 (597.7us) | 11.25 | 52.65 |
|  | 100 | 32,966 (1.10ms) | 36,014 (600.2us) | 36,966 (1.23ms) | 42,015 (700.3us) | 14.19 | 63.71 |
| 512-point | 24 | 26,408 (880.3us) | 28,753 (479.2us) | 29,360 (978.7us) | 33,182 (553.0us) | 2.23 | 9.46 |
|  | 48 | 47,588 (1.59ms) | 51,925 (865.4us) | 53,204 (1.77ms) | 60,350 (1.01ms) | 5.78 | 27.20 |
|  | 72 | 66,464 (2.22ms) | 72,577 (1.21ms) | 74,456 (2.48ms) | 84,566 (1.41ms) | 10.79 | 52.65 |
|  | 100 | 85,574 (2.85ms) | 93,486 (1.56ms) | 95,974 (3.20ms) | 109,087 (1.82ms) | 14.05 | 63.71 |
| 1024-point | 24 | 53,800 (1.79ms) | 58,577 (976.3us) | 59,824 (1.99ms) | 67,614 (1.13ms) | 2.21 | 9.46 |
|  | 48 | 99,556 (3.32ms) | 108,629 (1.81ms) | 111,316 (3.71ms) | 126,270 (2.10ms) | 5.85 | 27.20 |
|  | 72 | 143,008 (4.77ms) | 156,161 (2.60ms) | 160,216 (5.34ms) | 181,974 (3.03ms) | 10.69 | 52.65 |
|  | 100 | 190,790 (6.36ms) | 208,430 (3.47ms) | 213,990 (7.13ms) | 243,231 (4.05ms) | 13.99 | 63.71 |

*: Impulse Response.

| Table 3.2.1 - Benchmark of 32-bit FIR Filter: generic |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of Taps | I.R.* in SRAM (cycles) |  | I.R.* in FLASH (cycles) |  | Error |  |
|  |  |  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |
| 64-point | 24 | 13,984 (466.1us) | 14,365 (239.4us) | 16,608 (553.6us) | 18,624 (310.4us) | 2.10 | 12.40 |
|  | 48 | 11,101 (370.0us) | 11,419 (190.3us) | 13,311 (443.7us) | 14,865 (247.8us) | 2.50 | 14.40 |
| 256-point | 24 | 79,073 (2.64ms) | 81,181 (1.35ms) | 93,985 (3.13ms) | 105,408 (1.76ms) | 2.30 | 17.40 |
|  | 48 | 135,518 (4.52ms) | 139,291 (2.32ms) | 162,688 (5.42ms) | 181,713 (3.03ms) | 2.70 | 16.60 |
|  | 72 | 177,131 (5.90ms) | 182,137 (3.04ms) | 213,391 (7.11ms) | 238,002 (3.97ms) | 4.60 | 31.50 |
|  | 100 | 187,238 (6.24ms) | 194,313 (3.24ms) | 241,717 (8.06ms) | 264,808 (4.41ms) | 4.60 | 31.90 |
| 512-point | 24 | 165,857 (5.53ms) | 170,269 (2.84ms) | 197,153 (6.57ms) | 221,120 (3.69ms) | 2.60 | 23.10 |
|  | 48 | 301,406 (10.05ms) | 309,787 (5.16ms) | 361,856 (12.06ms) | 404,177 (6.74ms) | 3.20 | 23.90 |
|  | 72 | 422,123 (14.07ms) | 434,041 (7.23ms) | 508,559 (16.95ms) | 567,218 (9.45ms) | 5.10 | 35.30 |
|  | 100 | 492,390 (16.41ms) | 510,985 (8.52ms) | 635,701 (21.19ms) | 696,424 (11.61ms) | 5.10 | 31.90 |
| 1024-point | 24 | 339,425 (11.31ms) | 348,445 (5.81ms) | 403,489 (13.45ms) | 452,544 (7.54ms) | 3.70 | 28.40 |
|  | 48 | 633,182 (21.11ms) | 650,779 (10.85ms) | 760,192 (25.34ms) | 849,105 (14.15ms) | 4.60 | 29.30 |
|  | 72 | 912,107 (30.40ms) | 937,849 (15.63ms) | 1,098,895 (36.63ms) | 1,225,650 (20.43ms) | 6.50 | 35.30 |
|  | 100 | 1,102,694 (36.76ms) | 1,144,329 (19.07ms) | 1,423,669 (47.46ms) | 1,559,656 (26.00ms) | 6.40 | 33.30 |

*: Impulse Response.

| Table 3.2.2 - Benchmark of 32-bit FIR Filter: avr32-uc3 optimized |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of Taps | I.R.* in SRAM (cycles) |  | I.R.* in FLASH (cycles) |  | Error |  |
|  |  |  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz | 0 at 30MHz | 1 at 60MHz |  |  |
| 64-point | 24 | 6,479 (216.0us) | 6,613 (110.2us) | 9,141 (304.7us) | 10,918 (182.0us) | 2.10 | 12.40 |
|  | 48 | 5,132 (171.1us) | 5,245 (87.4us) | 7,305 (243.5us) | 8,866 (147.8us) | 2.50 | 14.40 |
| 256-point | 24 | 36,432 (1.21ms) | 37,140 (619.0us) | 51,574 (1.72ms) | 61,605 (1.03ms) | 2.30 | 12.40 |
|  | 48 | 62,157 (2.07ms) | 63,420 (1.06ms) | 88,906 (2.96ms) | 107,937 (1.80ms) | 2.70 | 16.60 |
|  | 72 | 81,114 (2.70ms) | 82,788 (1.38ms) | 116,446 (3.88ms) | 142,173 (2.37ms) | 4.60 | 31.50 |
|  | 100 | 94,441 (3.15ms) | 96,490 (1.61ms) | 140,910 (4.70ms) | 166,671 (2.78ms) | 4.60 | 31.90 |
| 512-point | 24 | 76,368 (2.55ms) | 77,844 (1.30ms) | 108,150 (3.61ms) | 129,189 (2.15ms) | 2.60 | 23.10 |
|  | 48 | 138,189 (4.61ms) | 140,988 (2.35ms) | 197,706 (6.59ms) | 240,033 (4.00ms) | 3.20 | 23.90 |
|  | 72 | 193,242 (6.44ms) | 197,220 (3.29ms) | 277,470 (9.25ms) | 338,781 (5.65ms) | 5.10 | 35.30 |
|  | 100 | 248,297 (8.28ms) | 253,674 (4.23ms) | 370,542 (12.35ms) | 438,287 (7.30ms) | 5.10 | 31.90 |
| 1024-point | 24 | 156,240 (5.21ms) | 159,252 (2.65ms) | 221,302 (7.38ms) | 264,357 (4.41ms) | 3.70 | 28.40 |
|  | 48 | 290,253 (9.68ms) | 296,124 (4.94ms) | 415,306 (13.84ms) | 504,225 (8.40ms) | 4.50 | 29.30 |
|  | 72 | 417,498 (13.92ms) | 426,084 (7.10ms) | 599,518 (19.99ms) | 731,997 (12.20ms) | 6.50 | 35.30 |
|  | 100 | 556,009 (18.53ms) | 568,042 (9.47ms) | 829,806 (27.66ms) | 981,519 (16.36ms) | 6.40 | 33.30 |

*: Impulse Response.

| Table 4.1.1 - Benchmark of 16-bit FIR Filter: generic |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Order of the filter | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 72-point | 2 | 5,294 (176.5us) | 5,717 (95.3us) | 1.50 | 4.00 |
|  | 7 | 11,469 (382.3us) | 12,802 (213.4us) | 1.40 | 4.30 |
|  | 12 | 16,319 (544.0us) | 18,387 (306.5us) | 2.50 | 7.60 |
| 256-point | 2 | 19,096 (636.5us) | 20,621 (343.7us) | 1.50 | 4.00 |
|  | 7 | 44,591 (1.49ms) | 49,786 (829.8us) | 1.40 | 4.30 |
|  | 12 | 68,761 (2.29ms) | 77,451 (1.29ms) | 3.50 | 8.90 |
| 512-point | 2 | 38,296 (1.28ms) | 41,357 (689.3us) | 1.50 | 4.00 |
|  | 7 | 90,671 (3.02ms) | 101,242 (1.69ms) | 1.40 | 4.30 |
|  | 12 | 141,721 (4.72ms) | 159,627 (2.66ms) | 3.70 | 8.90 |
| 1024-point | 2 | 76,696 (2.56ms) | 82,829 (1.38ms) | 1.50 | 4.00 |
|  | 7 | 182,831 (6.09ms) | 204,154 (3.40ms) | 1.40 | 4.30 |
|  | 12 | 287,641 (9.59ms) | 323,979 (5.40ms) | 3.90 | 8.90 |

| Table 4.1.2 - Benchmark of 16-bit FIR Filter: avr32-uc3 optimized |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Order of the filter | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-5) | Max. amplitude (x10-5) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 72-point | 2 | 2,725 (90.8us) | 2,871 (47.9us) | 1.20 | 4.00 |
|  | 7 | 4,332 (144.4us) | 4,682 (78.0us) | 1.90 | 6.90 |
|  | 12 | 6,089 (203.0us) | 6,534 (108.9us) | 3.30 | 8.40 |
| 256-point | 2 | 9,167 (305.6us) | 9,633 (160.6us) | 1.10 | 4.00 |
|  | 7 | 15,006 (500.2us) | 16,228 (270.5us) | 1.70 | 6.90 |
|  | 12 | 22,283 (742.8us) | 23,876 (397.9us) | 5.10 | 12.40 |
| 512-point | 2 | 18,127 (604.2us) | 19,041 (317.4us) | 1.10 | 4.00 |
|  | 7 | 29,854 (995.1us) | 32,292 (538.2us) | 1.70 | 6.90 |
|  | 12 | 44,811 (1.49ms) | 48,004 (800.1us) | 5.60 | 12.40 |
| 1024-point | 2 | 36,047 (1.20ms) | 37,857 (631.0us) | 1.10 | 4.00 |
|  | 7 | 59,550 (1.99ms) | 64,420 (1.07ms) | 1.70 | 6.90 |
|  | 12 | 89,867 (3.00ms) | 96,260 (1.60ms) | 5.80 | 12.40 |

| Table 4.2.2 - Benchmark of 32-bit FIR Filter: generic |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Order of the filter | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 72-point | 2 | 7,582 (252.7us) | 8,072 (134.5us) | 1.00 | 3.80 |
|  | 7 | 14,517 (483.9us) | 15,922 (265.4us) | 1.50 | 7.00 |
|  | 12 | 19,952 (665.1us) | 22,097 (368.3us) | 2.60 | 10.30 |
| 256-point | 2 | 27,456 (915.2us) | 29,232 (487.2us) | 0.80 | 3.80 |
|  | 7 | 56,471 (1.88ms) | 61,922 (1.03ms) | 1.50 | 7.00 |
|  | 12 | 83,986 (2.80ms) | 92,937 (1.55ms) | 1.50 | 10.30 |
| 512-point | 2 | 55,104 (1.84ms) | 58,672 (977.9us) | 0.80 | 3.80 |
|  | 7 | 114,839 (3.83ms) | 125,922 (2.10ms) | 1.50 | 7.00 |
|  | 12 | 173,074 (5.77ms) | 191,497 (3.19ms) | 1.40 | 10.30 |
| 1024-point | 2 | 110,400 (3.68ms) | 117,552 (1.96ms) | 0.80 | 3.80 |
|  | 7 | 231,575 (7.72ms) | 253,922 (4.23ms) | 1.50 | 7.00 |
|  | 12 | 351,250 (11.71ms) | 388,617 (6.48ms) | 1.30 | 10.30 |

| Table 4.2.1 - Benchmark of 32-bit FIR Filter: avr32-uc3 optimized |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Order of the filter | Execution time (cycles) |  | Error |  |
|  |  |  |  | Amplitude average (x10-9) | Max. amplitude (x10-9) |
|  |  | wait-state |  |  |  |
|  |  | 0 at 30MHz | 1 at 60MHz |  |  |
| 72-point | 2 | 5,297 (176.6us) | 5,662 (94.4us) | 1.00 | 3.80 |
|  | 7 | 8,859 (295.3us) | 9,233 (153.9us) | 1.50 | 7.00 |
|  | 12 | 11,670 (389.0us) | 12,239 (204.0us) | 3.00 | 10.30 |
| 256-point | 2 | 18,731 (624.4us) | 20,014 (333.6us) | 0.80 | 3.80 |
|  | 7 | 33,333 (1.11ms) | 34,625 (577.1us) | 1.50 | 7.00 |
|  | 12 | 46,816 (1.56ms) | 48,855 (814.3us) | 1.80 | 10.30 |
| 512-point | 2 | 37,419 (1.25ms) | 39,982 (666.4us) | 0.80 | 3.80 |
|  | 7 | 67,381 (2.25ms) | 69,953 (1.17ms) | 1.60 | 7.00 |
|  | 12 | 95,712 (3.19ms) | 99,799 (1.66ms) | 1.70 | 10.30 |
| 1024-point | 2 | 74,795 (2.49ms) | 79,918 (1.33ms) | 0.80 | 3.80 |
|  | 7 | 135,477 (4.52ms) | 140,609 (2.34ms) | 1.60 | 7.00 |
|  | 12 | 193,504 (6.45ms) | 201,687 (3.36ms) | 1.60 | 10.30 |